UNIVERSAL PROGRAMMER & TESTER POWER-100/EXPRO-60/80

README.DOC

**********

1. POWER-100 or EXPRO-60/80 has consolidated its with coloured pull-down menu

driver software into 2 diskettes,

Diskette POWER-100 or EXPRO-60/80 #1 consists of the following files:

DEVICE.LST  : POWER-100 or EXPRO-60/80 can program Device's list.

INSTALL.BAT : Installation batch file.

EXPROXXX.EXE: EXPRO-60/80 executable SELF-EXTRACTING file includes

main program, edit-driver ,disassembler files and utility

files.

POWERXXX.EXE: POWER-100 executable SELF-EXTRACTING file includes

main program, edit-driver ,disassembler files and utility

files.

README.DOC  : This document records the information of revision of

S/W and H/W.

Diskette POWER-100 or EXPRO-60/80 #2 consists of the following files:

EXPRODRV.EXE: EXPRO-60/80 executable SELF-EXTRACTING file includes

all device driver files.

POWERDRV.EXE: POWER-100 executable SELF-EXTRACTING file includes

all device driver files.

Note: EXPRO-60/80 software was released on Sept 1992, all EXPRO-60

software before this date is not compatible with present

software. Please do not mix them up.

2. To enhance EXPRO-60/80's software function and make it easier to

operate, great amendments were made on the software driver. In

addition to altering into coloured version, other changes are:

a) pull-down menu driver furnishes user with simple operating

environment

b) detailed status field display provides user with complete IC

programming information, work path and current working file

c) all functions can be operated under main-menu providing user

with continuous operating procedures

d) equipped with user definable Macrofile function and QUICKLY keys

which greatly simplifies user's operating procedures

3. INSTALLATION

This segment describes the method of installing an UNIVERSAL

programmer on an IBM PC/XT/AT/386/486 computer or compatible

running MS-DOS or PC-DOS.

It is assumed that the installer is familiar with the installation

of PC add-on cards and software in PCs running MS-DOS.

3.1 Host System Requirements

*IBM PC/XT/AT/386/486 or compatible PC.

*PC/386DX-33 or faster is recommended.

*Min. 640K bytes memory.

*Min. One floppy disk drive. A hard disk is preferable.

*Operation system :MS-DOS or PC-DOS ,version 3.3 or later.

3.2 Software Installation Procedures

Running the INSTALL.BAT to setup POWER-100's or EXPRO-60/80's S/W to

HARD disk.

Running POWER.EXE(for POWER-100) or EXPRO.EXE(for EXPRO-60/80) will

start the system when installation is completed.

4. About GANGs Adaptors using

ADP-GAL16V18,   ADP-GAL20V8,    ADP-CE16V8, ADP-CE20V8,  ADP-N3001,

ADP-PEEL18CV8,  ADP-PEEL22CV10, ADP-EP32,   ADP-EP40,    ADP-EP32A-PLCC,

ADP-EP32B-PLCC, ADP-EP40-PLCC,  ADP-51F,    ADP-48F.

The adaptors listed above are GANGs adaptors (can program 4 devices

simultaneously). These GANGs adaptors' driver program (all in Adaptor

#3 diskette) needs to be independently executed and cannot be called

through "POWER.EXE" or "EXPRO.EXE". Therefore, the GANGs adaptor driver

file does not need to be in the same directory as POWER.EXE or EXPRO.EXE

while non-GANGs adaptor driver file must be in the same directory as

POWER-100 or EXPRO.EXE to be conveniently called through POWER.EXE or

EXPRO.EXE.

5. Revision of EXPRO.EXE or POWER-100

Version V7.00: On Sept 1, 1992, EXPRO-80 hardware and software were

formally released. Software is compatible with

EXPRO-60 hardware and will automatically identify

between EXPRO-60 and EXPRO-80 hardware.

Due to the restriction of DOS, our software can only

allocate memory buffer to the maximum of 256K bytes

(2 Mbits), thus we have in the "RunFunc" menu,

"Copy file to EPROM" and "Read EPROM to file" these

two functions to solve the problem of insufficient

memory buffer.

Below is its operating procedure:

"Read EPROM to file" - Software will require you to

input filename, then display "Ready to Read (Y/<CR>)?".

If you press 'Y', software will read EPROM's content on

the textool and save it into diskette.

"Copy file to EPROM" - Software will require you to select

the file to be copied to the EPROM file. After selection,

software will inquire "Ready to program (Y/<CR>)?".

When 'Y' is pressed, software will read the file's

content and program it to EPROM.

Version V7.10: On Nov 1,1992, software was updated with function of

executing adaptor with driver program. If you happen

to wish to program a Device that requires an adaptor,

the right bottom corner of Status field will display

"ADAPTOR: adaptor's module name" to remind you the

need of an adaptor in order to program.

Version V7.13: On Nov 20, 1992, to simplify user's operating procedure,

"Macro setup" and "Run Macro" were included in Misc.'s menu.

Briefly, MACRO's function in 'Misc.' are (1) Macro setup

(2) Run Macro,

Step 1: After selecting "Macro setup", the bottom corner

of status field displays " [Macro Mode]:<F9> to

abort, <F10> to save". Every operating procedure

will be recorded by the software till you press

<F10>. Software will then require you to input

Macro filename, after entering the filename,

"[Macro Mode]:..." will then vanish.

Step 2: After selecting "Run Macro", software will

require you to input the Macro file to be

executed. "Macro processing" will appear on

the bottom corner of status field after selecting

Macro file till execution is completed.

Version V7.15: On Dec 17, 1992, for the convenience of user, software

was updated to classify IC Test into the following:

1. Logic IC test     : Test TTL74 , CMOS40/45 series

2. Dynamic RAM test  : Test Dynamic RAM

3. Static RAM test   : Test Static RAM

4. User vector editor: User can edit Vector and test

ICs that we did not include in

our Type. Modes selectable are

8, 14, 16, 20, 24, 28, 32 and 40

pins.

Version V7.22: On Mar 23, 1993, EXPRO /M=MACROfilename

If you want to run MACRO file after entering

EXPRO.EXE program, you could input "EXPRO /M=MARCOFILENAME"

at DOS prompt.

version V8.00 : May 31, 1993

EXPRO.EXE V8.00 has the following modifications:

a. Software V8.00 can be used with either SAC-101A, System Adaptor Card or

LINK-P1, Printer Port Adaptor Module to control EXPRO-60/80 hardware.

The software automatically detects the kind of interface used. To

avoid confusions with old versions, the programming driver files' file

extention name EXE has been changed to BIN.

b. V8.00 and later versions simplify Read/Program of high-capacity EPROM.

When user selects EPROM size larger than software allocated memory

buffer, software opens a file (EDBIN.MEM) with same EPROM size

automatically in the "Work path" to use file buffer when software

access EPROM's data.

Example:

Assumption: software allocated memory buffer space is 256K bytes

Work path: D:\EXPRO\

When programming 27C040 (512 Kbytes), allocated memory buffer becomes

insufficient. Software automatically opens a 512 Kbytes file

(D:\EXPRO\EDBIN.MEM). If file opening is successful, "EDBIN.MEM" will

appear after "Buffer address :" inside the status field. After this,

the user may change work path's directory and not affect EDBIN.MEM

access.

Upon exiting the main program, D:\EXPRO\EDBIN.MEM is recorded in

EXPRO.DAT for future use. As not to occupy disk space,

D:\EXPRO\EDBIN.MEM is deleted.

NOTE: For executing the above, take note of the following:

a. If the PC's memory RAM is sufficient, it is suggested that the

excess memory RAM be set into RAM DISK (refer to DOS manual).

This way, the maximum speed can be attained during Read/Program

EPROM and avoid hard disk Read/Write frequency.

b. If the PC has insufficient RAM, it is suggested that the biggest free

space of hard disk of faster access time be used as not to affect the

speed of Read/Program EPROM.

c. If the disk space is insufficient or Read/Write is not possible,

"Disk-Buffer access error. Refer to README.DOC" would appear on the

screen. At this point, determine if there is sufficient buffer to

access EPROM data based on the length of the "Buf start:","Buf end:"

and "Dev start:","Dev end:" inside  the status field. If buffer is

insufficient, solve the above problem on the disk before reentering

main program.

version V8.20 : May 15, 1994

POWER.EXE or EXPRO.EXE V8.20 has the following modifications:

Configuration Menu:

A. Memory Initialization: Initial the content of memory and disk-buffer.

"00H","FFH" can be toggled by pressing [ENTER] key.  After entering

Main program (POWER.EXE OR EXPRO.EXE), the set value will be used to

initialize the content of memory and disk-buffer.

B. Disk-Buffer Path: Set up disk number and path of the disk-buffer.

When the DOS allocated memory buffer is out of space, system will

automatically use the set value to create additional disk-buffer.

Fast speed RAM-Disk or Hard disk is recommended.

C. Self-testing [ON/OFF]: Self testing function [ENABLE/DISABLE].

When enabled, system will perform self-testing function on all

programming driver circuit before entering Main-menu.  When there is

hardware circuit error, system will warn the user with error messages

and run "SELFDIAG.EXE" to perform further testing and diagnosis.  This

procedure should only take 4-10 seconds, we highly recommend this

self-testing procedure. (For POWER-100 only)

[ON/OFF] can be toggled by pressing [ENTER] key.

D. IC inserted test [ON/OFF]: Test IC inserted position [ENABLE/DISABLE].

When enabled, system performs IC inserted position test before

programming the IC. This test is to prevent any damage to the IC

inserted.   Furthermore, to protect the hardware of the programmer

from being damaged by overcurrent, system warns the user with

"IC overcurrent" message when this situation occurs and discontinues

the IC programming process.(For POWER-100 only)

[ON/OFF] can be toggled by pressing [ENTER] key.

E. Auto sense [ON/OFF]:  Auto sensing function [ENABLE/DISABLE].

When enabled, after entering any "Runfunc-menu" function, system

automatically senses the socket of the programmer to check if there

is any IC placed in it.  If IC is placed, system automatically enter

"Runfunc" without the need of pressing "YES" key.  This function will

simplify and benefit large volume production.(For POWER-100 only)

Note: when this function is enabled, after entering any "Runfunc-menu"

function, the "BUSY" LED of the POWER-100 will be turned on.

[ON/OFF] can be toggled by pressing [ENTER] key.

<<< IMPORTANT AMEND !!! >>>

version V8.25 : April 30, 1994

The following mention is for POWER-100 only.

POWER-100 HARDWARE REVISION

Due to the drivers of different PC's printer port, We were forced

to make some  changes to the hardware of the POWER-100, We feel sorry

to ask you to also do the necessary changes  to your  POWER-100 system.

For those  of you who are using software version V8.20 and prior

version,   the following  procedures is necessary,

1. Open POWER-100 and take out U21 (Lattice GAL16V8 or AMD PALCE16V8)

from bottom board( PCB NO. : EXPRO-100 PCB-E0007 ).

2. Use the JEDEC file provided under "UTIL" directory--"POWER.JED"

to program to the U21.

3. After the programming step is completed ok, put the U21 back.

4. Make sure all the steps stated above is correctly done, continue

to use version V8.25 or later software.

version V8.25 : April 30, 1994

The following mention is for EXPRO-60/80 only.

LINK-P1 HARDWARE REVISION

Due to the drivers of different PC's printer port, We were forced

to make some  changes to the hardware of the LINK-P1, We feel sorry

to ask you to also do the necessary changes  to your  LINK-P1 system.

For those  of you who are using software version V8.20 and prior

version,   the following  procedures is necessary,

1. Open LINK-P1 and take out U2 (Lattice GAL16V8 or AMD PALCE16V8).

2. Use the JEDEC file provided under "UTIL" directory--"LINK-P1.JED"

to program to the U2.

3. After the programming step is completed ok, put the U2 back.

4. Make sure all the steps stated above is correctly done, continue

to use version V8.25 or later software.

6.New additional DEVICEs list

ALTERA:

EPC1064/V( for POWER-100 & EXPRO-80 only )

EPC1213  ( for POWER-100 & EXPRO-80 only )

#EPM7064LC44 ( for POWER-100 )                  DATE:04/07/94

*EPM7064LC68 ( for POWER-100 & EXPRO-80 only )  DATE:04/07/94

*EPM7064LC84 ( for POWER-100 & EXPRO-80 only )  DATE:04/07/94

AMD :

Am28F010A,Am28F020A,MACH120,MACH220

MACH130,MACH230                         DATE:16/9/94

ATMEL:

AT29C512,AT27C4096

AT89C52                                 DATE:21/10/94

ATF16V8B/L,ATF20V8B/L                   DATE:01/13/95

ATF22V10B/L                             DATE:02/21/95

AT89C1051,AT89C2051                     DATE:01/18/95

ATV750B/BL                              DATE:02/22/95

CATALYST:

CAT28C16V3

CYPRESS:

CY7C291A,CY7C292A,CY7C256

CY27H010                                DATE:07/10/94

PALCE16V8                               DATE:02/21/95

DALLAS:

DS1220AB/AD,DS1225AB/AD,DS1225D/DE,DS1225Y

DS1230Y/AB, DS1245Y/AB

DS87C520                                DATE:01/13/95

FUJITSU:

28F010                                  DATE:7/10/94

MBM27C4096                              DATE:21/10/94

HITACHI:

HN27C4000G                              DATE:12/29/94

ICT:

PA7140                                  DATE:02/21/95

INTEL:

8795BH,8798 ( for POWER-100 only )

NOTICE: The above ICs must be inserted backward in the textool.

���������͸                         �����V���͸

PIN-25Ķ ��   �� ��PIN-24            PIN-1Ķ ��   �� ��PIN-48

PIN-26Ķ ��   �� ��PIN-23            PIN-2Ķ ��   �� ��PIN-47

PIN-27Ķ ��   �� ��PIN-22            PIN-3Ķ ��   �� ��PIN-46

.  � 8795BH  �    .                  . �         � .

.  �   or    �    .                  . � 48-PINS � .

.  �  8798   �    .  Backward insert . � TEXTOOL � .

.  �         �    .  ==============> . �         � .

�         �                         �         �

PIN-46Ķ ��   �� ��PIN-3            PIN-22Ķ ��   �� ��PIN-27

PIN-47Ķ ��   �� ��PIN-2            PIN-23Ķ ��   �� ��PIN-26

PIN-48Ķ ��   �� ��PIN-1            PIN-24Ķ ��   �� ��PIN-25

�����^���;                         ���������;

ISSI:

IS93C46-3,IS93C56-3,IS28F010            DATE:11/24/94

LATTICE:

GAL6001B,GAL6002B                       DATE:02/23/95

MICROCHIP:

PIC16C84,PIC16C64(for POWER-100 only),PIC16C58

MITSUBISHI:

M5M28C64A

MIXC:

MX27L1000,MX27C1001,MX27C4100,MX27C4096

MX27L256,MX27L512,MX27L4000,MX27C4111   DATE:21/10/94

MOTOROLA:

MC68HC711D3-PLCC(for POWER-100 only)    DATE:11/09/94

NEC:

uPD28C64

uPD75P108B                              DATE:12/30/94

OKI:

MSM28C16A

PHILIPS:

87C504, 87C524,87C575,87C51FC           DATE:01/18/95

SII:

S-2840A,S-2812A/17A,S-2860A/64A

WINBOND:

W27F010,W29EE011,W27F512,W27E512,W27E010 DATE:04/07/95

SGS-THOMSON:

ST62E10/T10,ST62E20/T20,ST62E15/T15,ST62E25/T25

NOTICE:  The above ICs must be inserted backward in textool.

It mean IC's pin-1 is mapped to textool's pin-22

(at EXPRO-80) or pin-21 (at EXPRO-60).(POWER-100 no need)

M27C160                                 DATE:7/10/94

M27C801,M28C64C                         DATE:21/10/94

XILINX:

XC1736D,XC1765D,XC17128,XC17128D,XC17256D

NOTICE: You have to connect a resistor(1.5K ohm) between PIN2 and PIN8

of IC when you use EXPRO-60 H/W. Because the speed of

EXPRO-60's driver pins are too slow.

7. To program EPROM by way of EVEN/ODD ( with driver file EPP1024.BIN

version V7.05 for 1M to 8M or driver file EPP512.BIN version V7.02

for 2716 to 27512 ), you can operate as follows,

EX: PROGRAM 27C256

a. Modify zone

Buffer start : 0

Buffer end   : 3FFF

Device start : 0

When you press 'Even' in the program function, software will execute

program procedure and halt at EPROM address 3FFF.

b. Modify zone

Buffer start : 0

Buffer end   : 7FFF

Device start : 4000

When you press 'Odd' in the program function, software will execute

program procedure and halt at address 7FFF.

8. The function of the "PLD Vector Test" is to test the internal circuit

of the PLD.

Test procedures:

a. Vector Table must be built when designing the PLD equation.

(Refer to the usage of PLD Assemblers)

b. After Assembler, the Vector Table is included in the Jedec

file generated by the Assembler.

c. After using the programmer to program the PLD successfully,

perform the " PLD Vector Test".

d. Select correct Type by "T. Select Type" Sub-menu.

e. After entering function" L. Load Test Vector of JEDEC file"

download the JEDEC file to memory buffer.

f. Then, enter function "F. Function Test", or "O. Loop Test",

to test the internal Circuit of the PLD.

Note: "V. View Vector" function can be used to view the Vector downloaded.

"S. Set Vcc" function can be used to set the Voltage Vcc value for

testing the PLD.

9. if POWER-100 communicates error with PC's printer port, refer to following:

a. Before using POWER-100, please check the AC voltage (110 or 220 ACV)

switch which is located at base of the unit.

b. POWER-100 should not be connected to SAC-101A. If it happens, the

unit's bottom board's U21(GAL-16V8) will be damaged. If so, please

replace it with another GAL16V8. In the POWER-100 "Util" path,

there is power.jed which is fuse map of U21.

c. If the above steps are correct, then connect the POWER-100 to the PC's

printer port, turn on the power, execute power.exe, then the s/w will

search for the POWER-100 via the PC's printer port to see which I/O

port it has been installed and there's no need for the user to make the

seting.

d. If the problem still exists after the above steps, then disconnect the

POWER-100 from the printer port and test the printer port with a printer

to check if the printer can print. If it can't print, then the super

I/O card should faultily. If it prints, then it might be caused by the

interference coming from the super I/O card since some cards produce

interferences, therefore use another super I/O card to test and let us

know its brand for our information.

e. Please send us the POWER-100 for repair, if still it can't work.

--------------------------------------------------------------------

UPDATE INFORMATION FOR SOFTWARE VERSION 8.41

--------------------------------------------------------------------

DRIVER FILE       OLD V.    NEW V.         ADD ITEM

-------------------------------------------------------------------

EEP3B              8.18     8.19        ADD ATMEL      AT29C020

ATMEL      AT29LV020

HITACHI    HN58064

SEEP4              8.17     8.18        ADD MICROCHIP  24C32

24LC32

24LC65

SEEP1              8.18     8.18                       24C08

24LC08

GAL2               8.20     8.21        MODIFY FOR AMD PALCE20V8PLCC

"IC OVER CURRENT" PROBLEM.