EXFILE VERSION 2.01

-------------------

New features since Version 1.34

-------------------------------

An Exfile Binary Format file editor program, EBFEDIT, is included in the

Exfile package.  This may be run either from the 'Files' menu of Exfile,

or as a standalone program.

New features since Version 1.33

-------------------------------

When generating Motorola S-Record files from Exfile Binary Format files,

output file now has only one S0 record (at start of file) and one S9

record (at end of file) rather than multiple S0 and S9 pairs.

New features since Version 1.32

-------------------------------

Binary data files (with no header) may now be converted to Exfile Binary

Format files, and vice versa.

New features since Version 1.31

-------------------------------

'Insertion Check' available in 'Programmer' menu, and also performed

before all other programmer functions.  This enhancement only takes effect

from EF-PER firmware release E2.06 onwards.

File conversion operations speeded up by approximately 10%.

The term 'Exfile Binary Format' is now used as an unambiguous name for the

internal file format used by Exfile, rather than just 'Binary' as in

previous versions.

When creating a new file, a previously existing file with the same name is

no longer automatically overwritten.  Instead, a warning is generated and

the user has the option of aborting the operation.

Bug fix:

Programmer operations previously failed when a General Instruments device

was selected, although the devices could be handled by selecting an

equivalent part from another family.  The problem is now fixed.

New features since Version 1.25

-------------------------------

Erase implemented for Intel flash (ETOX) devices and SEEQ 5233 EEPROMs.

Note that standard EEPROMs can be reprogrammed without being erased.

A 'Quit' from Exfile now causes 4000 and 5000 series models to exit from

remote mode.

Both these enhancements will only take effect with new EF-PER firmware

releases (i.e. E2.02 or later).

EF-PER firmware release number is shown in the status window at the bottom

of the screen.

Handling of disk errors is much improved, the error message 'Out of disk

space' indicating a file creation error.

Bug fix:

Checksums of 40-pin devices were previously displayed incorrecty, the

value displayed being the checksum of the lower 8 bits of the data bus.

The correct value is now displayed, this being the sum of the checksums

of the two halves of the data bus taken separately.

New features since Version 1.24

-------------------------------

Menus of files now contain a 'More..' entry to permit access to further

files where there are too many files for one screen.

New features since Version 1.22

-------------------------------

Bug fixes:

i)   The number of Exfile Binary Format (EBF) files is no longer limited to 16;

i.e. 'Other File..' no longer disappears from the bottom of the menu of

files under certain circumstances.

ii)  If there should be a comms lock-up, e.g. when programming, Version 1.22

sometimes caused the PC to hang for several minutes.  This is now fixed.

New command line option:

Adding /Q on the command line when calling up Exfile will now result in

faster screen updates when used with CGA colour display adapters.

However some CGA adapters (most notably the original IBM CGA cards) will

also produce a flickering 'snow' effect on screen when this option is used.